TECHNICAL SPECIFICATIONS COMPLIANCE FORM

RUBBER-TIRE WHEEL LOADER
FOR THE
MID-CONNECTICUT WASTE PROCESSING FACILITY

The rubber tire loader required by the Contract Documents must be an 8.25-cubic yard, front-end wheel loader suitable for use in a severe trash-to-energy application. The loader must have protection for extreme waste operations.

On the table on the following pages, each bidder must indicate whether or not the rubber tire loader it would provide complies with each of the specified technical specifications. Bidders must mark (“X”) in the "Yes" column if their loader complies with a technical specification or in the "No" column if their loader does not comply with a technical specification. Additionally, each item shall be initialed by the bidder in the provided column. Failure to comply with this request may cause a bid to be rejected.

Each item marked as not in compliance with a technical specification must be explained on an additional page attached by the bidder and signed. The presence of items that are not in compliance with the technical specifications will not necessarily disqualify a bidder from consideration for the work. The explanation for an item not in compliance with a technical specification must be in sufficient detail to ensure that CRRA can effectively evaluate the bid against all other bidders.

| Category/Technical Specifications | Complies? |  |  |
| --- | --- | --- | --- |
|  | Yes | No | Initial |
| - Operating Weight |  |  |  |
| - 54,000 pounds, minimum |  |  |  |
| - Wheel Base |  |  |  |
| - 11 ft. 0 inches, minimum |  |  |  |
| - Static Tipping |  |  |  |
| - 34,000 lbs., minimum load straight |  |  |  |
| - Engine |  |  |  |
| - Minimum 250 SAE J1349 Net Horse Power, 4 stroke, turbo charged, liquid cooled, diesel engine |  |  |  |
| - Must meet EPA Tier 3 emissions compliance |  |  |  |
| - Emissions met without the use of exhaust gas recirculation |  |  |  |
| - Spin on type oil filter, engine to be filled with Mobil Delvac 1 |  |  |  |
| - Primary and secondary air filters and pre-cleaner |  |  |  |
| - 110V Block heater, ether starting aid |  |  |  |
| - Murphy shut-down system or equivalent for low oil pressure and coolant temperature |  |  |  |
| - Fuel system priming with electric priming pump |  |  |  |
| - Transmission |  |  |  |
| - Full planetary power shift with 4 forward and 4 reverse speeds |  |  |  |
| - Ability to manually or automatically shift from high and low |  |  |  |
| - Ability to lock out third and fourth gears |  |  |  |
| - Gear shift selector on left of steering column |  |  |  |
| - Right hand mounted switch for forward, neutral, and reverse |  |  |  |
| - Minimum transmission oil capacity of 11 gallons |  |  |  |
| - Brakes |  |  |  |
| - Four-wheel hydraulic braking system |  |  |  |
| - Brake wear measured without axle disassembly |  |  |  |
| - Cooling System |  |  |  |
| - Hydraulically on-demand driven fan with reversing function |  |  |  |
| - Fan to pass air through radiator and exhaust through engine enclosure |  |  |  |
| - Engine and cooling system enclosure to be 1-piece design |  |  |  |
| - Enclosure to be manufactured of synthetic composite material to prevent corrosion |  |  |  |
| - Hydraulic System |  |  |  |
| - Sealed with full flow filtering, load sensing design with a minimum of 80 gpm circulation |  |  |  |
| - Bucket Controls |  |  |  |
| - Separate lift and tilt bucket controls |  |  |  |
| - Electric over hydraulic controls |  |  |  |
| - Adjustable automatic kick-out for lift tilt functions |  |  |  |
| - Steering |  |  |  |
| - Center-point frame articulation |  |  |  |
| - Articulation joint must use double tapered roller bearings |  |  |  |
| - Hydraulic, load-sensing power steering |  |  |  |
| - Steering wheel with tilt and telescopic column |  |  |  |
| - Tires |  |  |  |
| - 26.5R25 L-5 Radial design foam filled |  |  |  |
| - Electrical |  |  |  |
| - 80-amp alternator, 24-volt direct electric starting |  |  |  |
| - Electronic monitoring system with a standard gage package |  |  |  |
| - Front and rear washer wipers |  |  |  |
| - 24-12 Converter with minimum of 8 amps. |  |  |  |
| - Cab |  |  |  |
| - ROPS cab to meet criteria SAE J1040 APR88 and ISO 3471-1994 criteria. |  |  |  |
| - FOPS to meet SAE J231 JAN81 and ISO 3449:1992 Level 2 |  |  |  |
| - Cab with doors and windows closed to meet requirements for operator and sound exposure limits according ISO 6396:1998 |  |  |  |
| - Lockable cab with same key as ignition switch |  |  |  |
| - Left side cab access door with handrails |  |  |  |
| - Ability to remove cab for service without breaching wiring harness |  |  |  |
| - Roof-mounted air conditioner |  |  |  |
| - Safety Equipment |  |  |  |
| - Equipped with back up alarm |  |  |  |
| - 4 Whelen model TIR6 500 series amber warning lights or equivalent |  |  |  |
| - Whelen lights cab-mounted or equivalent |  |  |  |
| - Internal and external rearview mirrors |  |  |  |
| - Arms/Frame |  |  |  |
| - Loader frame must be high lift design and relieved in way of bucket cylinder to prevent trash build up under cylinder. |  |  |  |
| - High Lift - minimum hinge pin height 15' (high lift package) for stacking and loading 13' trailers from the ground |  |  |  |
| - Z-bar type Linkage |  |  |  |
| - Breakout force minimum of 36,000 lbs |  |  |  |
| - Static tipping load straight minimum 34,000 lbs |  |  |  |
| - Operating Dimensions |  |  |  |
| - Ground clearance no less then 20" |  |  |  |
| - Overall height not to exceed 13' 2" |  |  |  |
| - Overall length not to exceed 31' 6" |  |  |  |
| - Overall width not to exceed 11' 6" |  |  |  |
| - Bucket |  |  |  |
| - 8.25-cubic yard trash and refuse bucket with spill plate |  |  |  |
| - Bolt on rubber cutting edge |  |  |  |
| - Bucket width 11'-6" maximum |  |  |  |
| - Differentials |  |  |  |
| - Self-locking (limit slip design) w/ inboard mounted brakes |  |  |  |
| - Axel oil capacity not less than 14.5 gallons |  |  |  |
| - Front axle fixed and rear axle must oscillate +/- 13 degrees of center pivot and feature lifetime lubrication without maintenance. |  |  |  |
| - Machine Guarding |  |  |  |
| - Axle seal guards |  |  |  |
| - Steering cylinder guards |  |  |  |
| - Transmission and engine crankcase guard |  |  |  |
| - Headlight and taillight guards |  |  |  |
| - Heavy duty entry ladder |  |  |  |
| - Fire Suppressant System |  |  |  |
| - A2030-2 Afex Dual Automatic Dry Chemical System with Linear Detection, Afex CMP Control Panel 12/24 volt or equivalent |  |  |  |
| - Manual release in cab area with pneumatic releasing |  |  |  |
| - Heating/AC |  |  |  |
| - Internal heating, defrosting and air conditioning, interior electric fan, roof mounted condenser coil, fan and filters |  |  |  |
| - Additional Equipment Requirements |  |  |  |
| - Locks for fuel and hydraulic tank and all access doors |  |  |  |
| - Adjustable suspension seat with seat belts |  |  |  |
| - Back up alarm |  |  |  |
| - Turn signals |  |  |  |
| - Exterior lighting for travel |  |  |  |
| - Front warning horn |  |  |  |
| - AM/FM radio |  |  |  |
| - Provision for 2-way radio |  |  |  |
| - Outside & inside mirrors - adjustable |  |  |  |
| - Interior lights |  |  |  |
| - Handrails RH side |  |  |  |
| - Batteries - maintenance free with disconnect switch |  |  |  |
| - Z-bar type linkage |  |  |  |
| - Automatic grease lubrication system |  |  |  |
| - Rebuild Capability |  |  |  |
| - Ability to perform a power-train rebuild that is certified by the manufacturer |  |  |  |
| - Ability to perform a complete machine rebuild that is certified by the manufacturer |  |  |  |
| - Rebuild documented procedure available to CRRA upon request |  |  |  |
| - Rebuilt machine carries a "same as new" warranty at no cost |  |  |  |
| - Rebuild qualifies for extended warranty options |  |  |  |
| - Rebuild includes engine dynamometer test and transmission and hydraulic pumps bench tests with proof of test results upon request |  |  |  |
| - Warranty |  |  |  |
| - Unit must carry a one year machine warranty. A copy of the warrantee must be attached to the bid. |  |  |  |
| - Unit must include a 3 year preventative maintenance program to include all scheduled filter and fluid changes as called out by the manufacture’s published schedule and to be performed at a minimum of every 250 hours. This is to include all related costs such as travel time and mileage. The copy of the proposed preventative maintenance plan shall be attached to the bid proposal. |  |  |  |
| - Manuals |  |  |  |
| - Three (3) operating and three (3) service manuals provided by supplier. Include all standard features not directly specified which the manufacturer normally offers as standard equipment and quote in the base price. |  |  |  |
| - Parts Supply |  |  |  |
| - Ability to supply 98% of parts within a 24-hour period. |  |  |  |
| - Machine Information System |  |  |  |
| - The ability to monitor remotely (e.g., from an off-site office) critical machine functions via GPS including machine location, hours, and fault codes. |  |  |  |