TECHNICAL SPECIFICATIONS COMPLIANCE FORM

MOBILE GRAPPLE-CRANE
FOR THE
MID-CONNECTICUT WASTE PROCESSING FACILITY

The mobile grapple-crane required by the Contract Documents must have a 17-foot material boom with at least a 0.75-cubic yard rotation and be suitable for use in a severe trash-to-energy application.

On the table on the following pages, each bidder must indicate whether or not the mobile grapple-crane it would provide complies with each of the specified technical specifications. Bidders must mark (“X”) in the "Yes" column if their mobile grapple-crane complies with a technical specification or in the "No" column if their mobile grapple-crane does not comply with a technical specification. Additionally, each item shall be initialed by the bidder in the provided column. Failure to comply with this request may cause a bid to be rejected.

Each item marked as not in compliance with a technical specification must be explained on an additional page attached by the bidder and signed. The presence of items that are not in compliance with the technical specifications will not necessarily disqualify a bidder from consideration for the work. The explanation for an item not in compliance with a technical specification must be in sufficient detail to ensure that CRRA can effectively evaluate the bid against all other bidders.

A Certificate of Origin must be furnished for the unit supplied pursuant to this RFB.

| Category/Technical Specifications | Complies? |  |  |
| --- | --- | --- | --- |
|  | Yes | No | Initial |
| - Operating Weight |  |  |  |
| - Approximate weight range of 39,000 to 42,000 pounds |  |  |  |
| - Wheel Base |  |  |  |
| - 8’ 2”, minimum |  |  |  |
| - BOOM/GRAPPLE |  |  |  |
| - Box-type design with large dimensioned sealed bearings |  |  |  |
| - Grapple open-close and rotate circuits installed |  |  |  |
| - Working cylinders with hydraulic end position damping |  |  |  |
| - Safety check valves (load holding valves) for boom and stick cylinders |  |  |  |
| - Material boom 17’, stick 13’ plus 31’ max reach |  |  |  |
| - Grapple at least 0.75-cubic yard with rotation, 4 or 5 tine, open, semi-closed or closed |  |  |  |
| - Engine |  |  |  |
| - 4-cylinder, water cooled, turbo diesel engine, with direct injection turbocharge and EPA Tier 3 emissions standard |  |  |  |
| - Output as per DIN/ISO: 114 HP at 2000 rpm or equivalent |  |  |  |
| - Transmission |  |  |  |
| - All-wheel drive via variable hydraulic motor with directly mounted automatic brake valve and 2-stage power-shift gear, or equivalent |  |  |  |
| - Maximum drawbar pull – 18,659 pound force |  |  |  |
| - SWING SYSTEM |  |  |  |
| - Axial piston motor, spring-loaded hydraulically releasable multiply disk brake, planetary gear and pinion shaft |  |  |  |
| - Large dimensioned swing bearing |  |  |  |
| - Swing speed 0 – 8 rpm, infinitely controllable |  |  |  |
| - Maximum torque – 29,500 pound-force foot or equivalent |  |  |  |
| - Hydraulic motor with integrated hydraulic brake |  |  |  |
| - Brakes |  |  |  |
| - Hydraulically activated single-circuit system that works on all 4 pair of wheels |  |  |  |
| - Hydraulic or electrical activated spring applied disk brake on the drive transmission for front and rear axles |  |  |  |
| - Cooling System |  |  |  |
| - Hydraulically driven and reversible radiator fan for clean-out |  |  |  |
| - Fan to pass air through radiator and exhaust through engine enclosure |  |  |  |
| - Equipped with engine intercooler |  |  |  |
| - Dry air filter with cyclone pre-filter, safety element and pollution control |  |  |  |
| - Hydraulic System |  |  |  |
| - Sealed with full flow filtering, load sensing design with 77 gpm |  |  |  |
| - Working pressure 5200 psi |  |  |  |
| - Hydraulic tank capacity 66 gallons or equivalent |  |  |  |
| - Ultra-fine filtration system (99% efficient) to protect hydraulics |  |  |  |
| - Computer free load sensing hydraulic system with energy saving flow, on demand control and pressure cut-off function |  |  |  |
| - Centralized pressure test ports |  |  |  |
| - Independent over-sized oil cooler |  |  |  |
| - UNDERCARRIAGE |  |  |  |
| - Undercarriage welded as rigid, torsion-free box-type design |  |  |  |
| - 4-point outriggers at approximately 13” between outer edge |  |  |  |
| - Lockable storage compartment |  |  |  |
| - All-wheel drive via variable hydraulic motor with directly mounted automatic brake valve and 2-state power-shift gear, or equivalent |  |  |  |
| - Heavy duty, extra wide, 36t axels |  |  |  |
| - Steering axle as hydraulically controllable oscillating axel |  |  |  |
| - Oscillating axle cylinders with safety check valves |  |  |  |
| - UPPERCARRIAGE |  |  |  |
| - Torsion-free upper frame with continuous bearing-plates precision machined |  |  |  |
| - Steel bushings for the boom pivot |  |  |  |
| - Separate radiator system for ambient temperatures, up to 122 degrees |  |  |  |
| - Tires |  |  |  |
| - Solid rubber elastic 8x9.00-20, or equivalent |  |  |  |
| - Electrical |  |  |  |
| - 80-amp alternator, 24-volt direct electric starting |  |  |  |
| - Electronic monitoring system with a standard gage package |  |  |  |
| - 12 and 24 volt power with minimum of 8 amps |  |  |  |
| - Windshield wiper/washer system |  |  |  |
| - Cab |  |  |  |
| - Hydraulic elevating cab with operator’s eye level 17’ minimum |  |  |  |
| - ROPS cab to meet criteria SAE J1040 APR88 and ISO 3471-1994 criteria |  |  |  |
| - FOPS to meet SAE J231 JAN81 and ISO 3449:1992 Level 2 |  |  |  |
| - Cab with doors and windows closed to meet requirements for operator and sound exposure limits according ISO 6396:1998 |  |  |  |
| - Lockable cab with same key as ignition switch |  |  |  |
| - Diagnostic system for monitoring essential machine and engine functions with visual and audible warning system |  |  |  |
| - Full instrument panel with ergonomically shaped joy stick |  |  |  |
| - Adjustable steering column |  |  |  |
| - Safety Equipment |  |  |  |
| - Machine must be equipped with back up alarm |  |  |  |
| - 4 Whelen model TIR6 500 series amber warning lights or equivalent |  |  |  |
| - Internal and external rearview mirrors |  |  |  |
| - Glare-free instrument cluster |  |  |  |
| - Warning system for operational conditions |  |  |  |
| - Operating Dimensions |  |  |  |
| - Ground clearance no less than 14" |  |  |  |
| - Overall cab height not to exceed 13' 2" |  |  |  |
| - Overall length with boom down not to exceed 31' 4" |  |  |  |
| - Overall width with riggers not to exceed 13' 0" |  |  |  |
| - MACHINE GUARDING |  |  |  |
| - Axel seal guards |  |  |  |
| - Steering cylinder guards |  |  |  |
| - Transmission and engine crankcase guard |  |  |  |
| - Headlight and taillight guards, if applicable |  |  |  |
| - Heavy duty entry ladder |  |  |  |
| - Fire Suppressant System |  |  |  |
| - A2030-2 Afex Dual Automatic Dry Chemical System with Linear Detection, Afex CMP Control Panel 12/24 volt |  |  |  |
| - Manual release in cab area with pneumatic releasing |  |  |  |
| - Heating/AC |  |  |  |
| - Internal heating, defrosting and air conditioning, interior electric fan |  |  |  |
| - Additional Equipment Requirements |  |  |  |
| - Locks for fuel and hydraulic tank and all access doors |  |  |  |
| - Adjustable suspension seat with seat belts |  |  |  |
| - Back-up alarm |  |  |  |
| - Turn signals |  |  |  |
| - Exterior lighting for travel |  |  |  |
| - Front warning horn |  |  |  |
| - AM/FM radio |  |  |  |
| - Provision for 2-way radio |  |  |  |
| - Outside & inside mirrors - adjustable |  |  |  |
| - Interior lights |  |  |  |
| - Handrails |  |  |  |
| - Batteries - maintenance free with disconnect switch |  |  |  |
| - Provide Z-bar type linkage |  |  |  |
| - Automatic grease lubrication system |  |  |  |
| - Sound level – machine sound poser Lwa = 100 dB(A) Max. |  |  |  |
| - Sound pressure level – driver’s seat Lpa = 71 dB(A) Max. |  |  |  |
| - Rebuild Capability |  |  |  |
| - Bidder shall have the ability to perform a complete machine rebuild that is certified by the manufacturer |  |  |  |
| - Rebuild should have documented procedure available to CRRA upon request |  |  |  |
| - Rebuilt machine carries a "same as new" warranty at no cost |  |  |  |
| - Rebuild must qualify for extended warranty options |  |  |  |
| - Rebuild shall include engine dynamometer test and transmission and hydraulic pumps must be bench tested with proof of test results upon request |  |  |  |
| - Warranty |  |  |  |
| - 24 months, 4000 hours – full machine warranty 36 months, 6000 hours – power-train warranty 60 months, 10,000 hours – structural warranty |  |  |  |
| - Oil sampling provided at no charge during the warranty period; oil sample must be analyzed by OEM certified lab; written qualifications of factory approval to be provided with bid |  |  |  |
| - Unit must include a 3-year preventative maintenance program to include all scheduled filter and fluid changes as called out by the manufacture’s published schedule and to be performed at a minimum of every 250 hours. This is to include all related costs such as travel time and mileage. The copy of the proposed preventative maintenance plan shall be attached to the bid. |  |  |  |
| - Manuals |  |  |  |
| - Three (3) operating and three (3) service manuals provided by supplier. Include all standard features not directly specified which the manufacturer normally offers as standard equipment and quote in the base price. |  |  |  |
| - Machine Information System |  |  |  |
| - The ability to monitor from an office critical machine functions via GPS including machine location, hours, and fault codes or an equivalent system |  |  |  |
| - PARTS SUPPLY |  |  |  |
| - The ability to supply 98% of parts within a 24-hour period |  |  |  |